UDC: 615.453.6

IRSTI: 61.45.15

QUALITY RISK ASSESSMENT OF A DIRECT COMPRESSION TECHNOLOGY FOR LOZENGES CONTAINING AJANIA FRUTICULOSA (LEDEB.) POLJAK EXTRACT WITHIN THE QUALITY BY DESIGN FRAMEWORK

A.M. Jakiyanov1, Z.B. Sakipova1, Z.B. Allambergenova1, A. Kadyrbay2, R.I. Kulmanbetov1, G.S. Ibadullayeva1, M.K. Suleimenov1*

¹ S.D. Asfendiyarov Kazakh National Medical University, Kazakhstan, Almaty

² Viva Pharm LLP, Kazakhstan, Almaty

*Corresponding author

Abstract

Background. Direct compression is a technologically and economically advantageous method for manufacturing lozenges, as it eliminates wet granulation and drying stages. However, the inherent variability of the physicochemical and technological properties of plant-derived substances may increase the probability of quality-related risks and compromise the reproducibility of critical quality attributes. Therefore, the implementation of a risk-based approach in accordance with the International Council for Harmonisation guideline Q9(R1), within the Quality by Design framework, is essential for the systematic identification of variability sources and the scientific justification of an appropriate control strategy.

Objective. To identify and assess quality-related risks associated with the development of a direct compression technology for lozenges containing dry Ajania fruticulosa extract within the QbD framework.

Materials and methods. Potential sources of variability were systematically identified and structured using an Ishikawa cause-and-effect diagram. Quantitative risk assessment was performed using Failure Mode and Effects Analysis. Severity, occurrence, and detectability were evaluated using a five-point scale, followed by calculation of the Risk Priority Number. Residual risk was assessed after modelling the proposed risk mitigation measures.

Results. Seven priority failure modes were identified. The initial Risk Priority Number values ranged from 18 to 48. The highest value was assigned to segregation of the powder blend during feeding into the hopper zone. The proposed risk mitigation measures targeting critical material attributes and critical process parameters were predicted to reduce residual Risk Priority Number values to 6–24. Based on residual risk assessment, six of the seven failure modes were transferred to the low-risk zone.

Conclusions. The proposed process control strategy provides a scientifically justified basis for the preparation of regulatory documentation, technology transfer, and scale-up of the direct compression technology for lozenges containing dry Ajania fruticulosa extract.

Keywords: lozenges, Ajania fruticulosa, Quality by design, quality risk management, Ishikawa diagram, Failure Mode and Effects Analysis.

Introduction

The development of solid dosage forms requires a scientifically grounded and systematically organized approach that integrates formulation design, manufacturing process development, and quality control strategy. In contemporary pharmaceutical development, the Quality by Design (hereinafter – QbD) paradigm provides a structured framework in which product quality is regarded not merely as the result of end-product testing, but as the outcome of a comprehensive understanding of the formulation, manufacturing process, and sources of variability. Within this framework, the relationships between critical material attributes (hereinafter – CMAs), critical process parameters (hereinafter – CPPs), and critical quality attributes (hereinafter – CQAs) of the finished dosage form should be identified during the early stages of development and managed throughout the product lifecycle [1–3].

Quality risk management is a central element of QbD-based pharmaceutical development, as it enables systematic identification, evaluation, and control of factors that may affect product quality. According to the International Council for Harmonization (hereinafter – ICH) guideline Q9(R1), quality risk management comprises a systematic process for the assessment, control, communication, and review of risks to the quality of medicinal products. In this context, risk assessment tools are used to determine which material attributes and process parameters are most likely to influence CQAs and therefore require further investigation, monitoring, or control [2]. Cause-and-effect analysis and Failure Mode and Effects Analysis (hereinafter – FMEA) are among the most applied tools for this purpose, as they allow potential sources of variability to be structured, their impact to be evaluated, and appropriate control strategies to be developed during formulation and process development [4–8].

Direct compression is widely used for the manufacture of tablets and lozenges because of its technological simplicity, reduced number of unit operations, and omission of wet granulation and drying stages. These advantages make the method attractive from both technological and economic perspectives. However, the simplified nature of the process also increases its dependence on the intrinsic properties of the powder blend, since there are fewer intermediate processing steps capable of correcting suboptimal material behavior. Consequently, powder flowability, compressibility, particle-size distribution, moisture content, and segregation tendency may directly affect key CQAs of the finished dosage form, including mass uniformity, content uniformity, resistance to crushing, friability, and disintegration behavior [8, 9, 10].

The need for a risk-based development strategy becomes particularly important when direct compression is applied to formulations containing active pharmaceutical ingredients (APIs) of plant origin. Unlike many synthetic pharmaceutical substances, herbal extracts are complex multicomponent systems characterized by natural variability in chemical composition, hygroscopicity, particle morphology, particle-size distribution, and technological performance [11]. These properties may adversely affect powder flow, compaction behavior, and process reproducibility, thereby increasing the likelihood of variability in the quality of the finished dosage form [11, 12].

In this regard, Ajania fruticulosa (Ledeb.) Poljak represents a promising plant-derived substance for further pharmaceutical development. This perennial species of the Asteraceae family is widely distributed in Kazakhstan, other regions of Central Asia, Mongolia and China. Phytochemical investigations of its aerial parts have reported the presence of flavonoids, terpenoids, phenolic acids, and sesquiterpenoid compounds, which supports its potential as a source of biologically active constituents for pharmaceutical applications [13–15]. Based on this phytochemical potential, previous formulation studies established the composition and preliminary technological parameters of lozenges containing dry Ajania fruticulosa extract using the QbD approach [13].

Nevertheless, although the formulation composition and preliminary technological parameters of these lozenges have been previously proposed, the quality-related risks associated with their manufacture by direct compression have not yet been sufficiently systematized. In particular, the material- and process-related factors that may affect the CQAs of the dosage form during formulation development, technology transfer, and scale-up require further identification, assessment, and prioritization. Such an assessment is necessary to support the development of a scientifically justified control strategy and to improve the robustness and reproducibility of the manufacturing process.

Therefore, the aim of the present study was to identify and assess quality-related risks associated with the development of a direct compression technology for lozenges containing dry Ajania fruticulosa extract within the QbD framework.

Materials and Methods

This study was designed as an experimental pharmaceutical development study incorporating a structured quality risk assessment. The object of the study was the direct compression technology for lozenges containing dry Ajania fruticulosa extract. The methodological framework included the identification of potential sources of variability, ranking of quality-related risks, and assessment of residual risk.

Manufacturing of lozenges by direct compression

Lozenges were manufactured by direct compression according to the procedure disclosed in Republic of Kazakhstan utility-model patent No. 11229, dated 3 October 2025 [16]. Dry Ajania fruticulosa extract was used as the API at a content of 3.75 mg per dosage unit. Sorbitol, copovidone, talc, magnesium stearate, and peppermint oil were used as excipients. Prior to processing, all solid components were passed through a 0.5 mm sieve. The API was initially blended with a portion of sorbitol until visual homogeneity was achieved. The remaining sorbitol, copovidone, peppermint oil, and talc were then added sequentially, while magnesium stearate was incorporated at the final blending stage. Final mixing was performed for 5–10 min at 30–40 °C, and the moisture content of the powder blend was maintained within the range of 2.5–3.0%. Compression was carried out using a single-punch tablet press at a compression force of 15–30 kN to obtain flat cylindrical lozenges with an average mass of 250 mg. The reported parameter ranges were considered preliminary operating ranges. Their formal establishment as a Design Space using Design of Experiments was beyond the scope of the present study.

Risk assessment

Initial identification of potential risk sources was performed using cause-and-effect analysis and presented in the form of an Ishikawa diagram. The identified factors were classified into seven categories: materials, direct compression process, equipment, quality control, personnel, environmental conditions, and packaging. In total, 36 potential factors were initially identified across these categories [2].

Factors were retained for detailed FMEA ranking if they met at least one of the following criteria: direct potential impact on predefined CQAs, including uniformity of content, uniformity of mass, friability, resistance to crushing, and disintegration, association with the known technological variability of herbal extracts, relevance to the robustness of the direct compression process, or anticipated need for additional control during scale-up. Factors considered to have only indirect or minor influence were retained within the general control strategy but were not subjected to detailed Risk Priority Number (hereinafter – RPN) ranking [2,17].

FMEA was selected as the quantitative risk-ranking tool because the study required a systematic comparison of multiple material- and process-related risk factors that may affect the quality of lozenges manufactured by direct compression. Unlike fault tree analysis, which is primarily used to investigate a defined failure event, and unlike Hazard Analysis and Critical Control Point (HACCP), which is mainly focused on the identification and control of critical control points, FMEA allows heterogeneous failure modes to be evaluated simultaneously according to several independent criteria [2, 17]. These criteria include severity of impact on CQAs (hereinafter – S), probability of occurrence (hereinafter – O), and probability of detection (hereinafter – D).

The use of FMEA was therefore considered methodologically appropriate for this study because the manufacturing process involved a plant-derived extract with variable technological properties. Such variability may influence powder flow, compressibility, moisture sensitivity, and reproducibility of compression. FMEA enabled the transformation of qualitatively identified risk factors into a quantitative ranking system and provided a basis for prioritizing proposed risk mitigation measures. The sequential use of Ishikawa analysis and FMEA was applied to combine qualitative root-cause structuring with quantitative risk prioritization within the QbD framework.

For each potential failure mode, the possible consequences for CQAs, the most probable causes, and the existing control measures were identified. Risk was assessed using three criteria: S, O, and D. Each criterion was scored on a five-point scale, as presented in Table 1. Scoring was performed by an expert panel consisting of five specialists representing the academic and industrial partners of the study, namely the School of Pharmacy at KazNMU and Viva Pharm LLP. The panel members had combined expertise in formulation development, pharmaceutical technology, and quality control.

Panel members were selected based on their professional background in one of these areas and a minimum of five years of relevant practical experience. Three panel members were also co-authors of the manuscript and contributed respectively to scientific supervision, risk identification and scoring, and evaluation of the technological feasibility of the proposed risk mitigation measures from a manufacturing perspective. Initial scores were assigned independently and subsequently harmonized by consensus after discussion of the technological relevance of each failure mode, its expected impact on CQAs, and the ability of existing in-process controls to detect the failure before batch release. If consensus could not be reached, the higher score was retained as a conservative measure.

The five-point scales for severity, occurrence, and detectability were adapted from the illustrative S/O/D scoring approach presented in ICH Q9(R1) and from the revised risk-matrix structure proposed by Ccopa Ibarra et al. [18]. The level descriptors were reformulated to reflect the CQAs relevant to direct-compression lozenge manufacturing, specifically uniformity of content, uniformity of mass, friability, resistance to crushing, and disintegration, rather than the generic process descriptors provided in the source schemes.

The Risk Priority Number was calculated according to the following equation:

RPN=S×O×D
Risks were ranked according to their RPN values, and the need for proposed risk mitigation measures was determined in accordance with the classification presented in Table 2.

The effectiveness of the proposed risk mitigation measures was assessed by re-rating O and D after introduction of the corresponding measures and by recalculating the RPN to determine residual risk. The severity score was kept constant for all failure modes, based on the methodological assumption that severity reflects the intrinsic impact of a failure on the affected CQA and is not altered by procedural or technological controls. The recalculated RPN values therefore represent expert-predicted residual risks expected after implementation of the proposed risk mitigation measures, rather than experimentally confirmed process-validation outcomes. Quantitative confirmation of mitigation effectiveness through direct CQA measurement was defined as a subsequent stage of process development

Table 1. Severity, occurrence and detectability scales used in FMEA

| Score | S – Impact on CQA | O – Probability of failure | D – Likelihood of detection |
| --- | --- | --- | --- |
| 1 | No impact on CQA | Highly unlikely | Deviation is easily detected |
| 2 | Minor impact on a single CQA | Rare | High probability of detection |
| 3 | Moderate impact, with possible deviations from acceptance criteria | Possible | Moderate probability of detection |
| 4 | Significant impact requiring rigorous in-process control | Highly probable and may occur frequently under insufficient CPP regulation, limited control barriers, and variable environmental conditions | Low probability of detection |
| 5 | Critical impact resulting in non-compliance | Almost inevitable and occurs systematically until formulation or process parameters are modified | Practically undetectable |

Source: compiled by the authors

Table 2. Interpretation of Risk Priority Number and management actions

| Risk zone | RPN range | Interpretation | Management action |
| --- | --- | --- | --- |
| Low | 1–20 | Risk is acceptable | Maintain existing control points and continue parameter monitoring |
| Medium | 21–50 | Risk requires management | Refine CMAs and CPPs, narrow tolerances, strengthen operational control, and conduct additional confirmatory testing where necessary |
| High | 51–125 | Risk is unacceptable | Propose risk mitigation measures, modify formulation or process parameters where necessary, revise the control strategy, and reassess RPN |

Source: compiled by the authors

Ethical considerations

The study did not involve human participants, personal medical data or laboratory animals. Therefore, approval by an ethics committee was not required.

Results

The Ishikawa cause-and-effect analysis enabled the systematic identification of potential sources of variability during the development of lozenges containing dry Ajania fruticulosa extract by direct compression. A total of 36 potential factors were identified and grouped into seven categories: materials, direct compression process, equipment, quality control, personnel, environmental conditions, and packaging (Figure 1). Material-related factors included the technological properties of the API and excipients. The main CMAs of the dry Ajania fruticulosa extract were particle-size distribution, flowability, compressibility, hygroscopicity, and solubility. Sorbitol, copovidone, talc, and magnesium stearate were also identified as potential sources of variability due to their influence on powder flow, interparticle interactions, adhesion behavior, lubrication efficiency, and compression performance.

Process-related factors were mainly associated with blending and compression. The CPPs included blending time, mixing intensity, component addition sequence, intermediate holding time of the powder blend, compression force, and mass-feed conditions. Equipment-related factors included blender geometry, technical condition of the tablet press, die and punch condition, sieve wear, and weighing accuracy. Environmental temperature and relative humidity were identified as additional factors because of their potential influence on moisture content of the powder blend and reproducibility of compression.

Based on the Ishikawa analysis, seven priority failure modes were selected for detailed FMEA ranking. Selection was based on their direct potential impact on CQAs, relevance to the robustness of direct compression, and expected importance during technology transfer and scale-up. The selected failure modes were associated with uniformity of content, uniformity of mass, friability, resistance to crushing, and disintegration.

Figure 1. Ishikawa diagram for the identification of quality-risk sources during the development of lozenges by direct compression.

Source: compiled by the authors

Initial RPN values for the selected failure modes ranged from 18 to 48 (Table 3). The highest initial RPN was observed for segregation of the powder blend during feeding into the hopper zone, which was rated at 48. Non-uniform distribution of the API in the powder blend and increased moisture content of the powder blend each exhibited an RPN of 36. Increased adhesion of the powder blend to blender walls and increased lozenge friability were both quantified at 27. Variation in lozenge mass was assigned an RPN of 24. Insufficient resistance to crushing showed the lowest initial RPN among the selected priority failure modes, with a value of 18.

According to the predefined risk classification, six of the seven priority failure modes were assigned to the medium-risk zone before consideration of the proposed risk mitigation measures. One failure mode, insufficient resistance to crushing, was classified as low risk. No high-risk failure modes were identified.

Proposed risk mitigation measures were formulated for all selected priority failure modes. These measures included standardization of blending parameters, control of particle-size distribution, regulation of moisture content, optimization of the component addition sequence, refinement of compression parameters, regulation of mass-feed conditions, and introduction of additional in-process control points. After re-rating occurrence and detectability under the proposed mitigation scenario, residual RPN values decreased for all evaluated failure modes and ranged from 6 to 24.

The residual risk assessment showed that six of the seven priority failure modes were transferred to the low-risk zone. Segregation of the powder blend during feeding into the hopper zone remained in the medium-risk zone, with a residual RPN of 24, and therefore required continued in-process monitoring. The complete distribution of S, O, D, and RPN values before and after application of the proposed risk mitigation measures is presented in Table 3.

Table 3. Risk assessment of the technological stages of lozenge manufacturing by direct compression.

| № | Process stage | Failure mode | Affected CQA | Initial S/O/D/RPN | Proposed risk mitigation measures | Residual S/O/D/RPN | Residual risk zone |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Preparation of powder blend for compression | Segregation of the powder blend during feeding into the hopper zone | Uniformity of content | 4 / 4 / 3 / 48 | Standardization of blending parameters, control of particle-size distribution, moisture content, and intermediate holding time of the powder blend | 4 / 3 / 2 / 24 | Medium risk, continued in-process monitoring required |
| 2 | Blending | Non-uniform distribution of the API in the powder blend | Uniformity of content | 4 / 3 / 3 / 36 | Optimization of the component addition sequence, regulation of blending time, and control of uniformity of content | 4 / 3 / 1 / 12 | Low risk, routine monitoring |
| 3 | Blending | Increased adhesion of the powder blend to blender walls | Uniformity of content | 3 / 3 / 3 / 27 | Moisture control and use of glidants at the optimal concentration | 3 / 1 / 3 / 9 | Low risk, routine monitoring |
| 4 | Preparation of powder blend | Increased moisture content of the powder blend | Disintegration, friability, uniformity of content | 3 / 4 / 3 / 36 | Introduction of a moisture-content control point and restriction of the CPP range | 3 / 2 / 2 / 12 | Low risk, routine monitoring |
| 5 | Compression | Increased lozenge friability | Friability | 3 / 3 / 3 / 27 | Standardization of compression parameters and control of lozenge surface quality | 3 / 3 / 1 / 9 | Low risk, routine monitoring |
| 6 | Compression | Variation in lozenge mass | Uniformity of mass, dosage unit uniformity | 4 / 3 / 2 / 24 | Regulation of mass-feed parameters and periodic control of lozenge mass | 4 / 2 / 1 / 8 | Low risk, routine monitoring |
| 7 | Compression | Insufficient resistance to crushing | Resistance to crushing, friability | 3 / 3 / 2 / 18 | Optimization of compression force and control of binder content | 3 / 2 / 1 / 6 | Low risk, routine monitoring |

Note. The values were obtained by expert FMEA scoring. Residual RPN values represent expert-predicted risk levels after consideration of the proposed risk mitigation measures rather than experimentally validated process outcomes.

Source: compiled by the authors based on Ishikawa analysis and FMEA scoring.

Discussion

The present study established a quality-risk profile for lozenges containing dry Ajania fruticulosa extract manufactured by direct compression. The highest initial RPN was associated with segregation of the powder blend during feeding into the hopper zone, while non-uniform API distribution and increased moisture content were also identified as priority risks. These findings indicate that the main sources of variability were concentrated at the stages of blending, intermediate handling, feeding, and compression.

This risk hierarchy reflects the technological sensitivity of direct compression. The method requires stable flowability, compressibility, particle-size distribution, moisture content, and blend homogeneity. In the present formulation, these factors were especially important because the dosage form contained a low amount of dry Ajania fruticulosa extract, 3.75 mg per dosage unit. Therefore, even moderate segregation or local variability in the powder blend may affect uniformity of content.

The predominance of segregation-related risk can be explained by the multicomponent composition of the formulation and possible differences in particle size, density, morphology, and flow properties between the herbal API and excipients. Since segregation may occur after blending, it cannot be reliably corrected by final product testing alone. This explains why this failure mode remained a medium residual risk after consideration of the proposed risk mitigation measures and requires continued monitoring during scale-up.

The present results differ from those reported by Kaljević et al. [8], where FMEA was applied to compression-coated tablets with pulsatile carvedilol release. In that study, the main risks were associated with formulation variables affecting the release profile. In contrast, the highest-ranked risks in the present study were related to raw-material variability, powder blend behavior, and feeding reproducibility. This difference is mainly due to the nature of the API, since carvedilol is a well-characterized synthetic substance, whereas dry Ajania fruticulosa extract is a plant-derived material with more variable technological properties.

A similar distinction is observed in comparison with Naman et al. [12], who developed flavored paracetamol oral disintegrating mini-tablets using a risk-based QbD approach. Their risk assessment mainly focused on excipient selection and optimization of formulation and process variables. In the present work, the main risks were more closely related to the herbal API itself, particularly its influence on powder flow, moisture sensitivity, blend uniformity, and feeding behavior.

The increased risk associated with the herbal API is consistent with the technological limitations of plant-derived extracts. Such materials may vary in residual moisture, hygroscopicity, particle morphology, particle-size distribution, bulk density, and adhesion tendency. These properties can impair flowability and compressibility, increase adhesion to equipment surfaces, and promote segregation. As a result, material-related and blending-related failure modes occupied the highest positions in the risk hierarchy.

The proposed risk mitigation measures reduced predicted RPN values for all selected failure modes. The greatest reductions were observed for non-uniform API distribution, increased moisture content, and variation in lozenge mass. These reductions were associated with optimization of component addition sequence, regulation of blending time, moisture-content control, refinement of mass-feed conditions, and additional in-process controls. However, segregation during feeding remained a medium residual risk, indicating that procedural controls alone may not fully eliminate this failure mode.

Compared with previous studies on directly compressed herbal tablets, which mainly focused on formulation development and product characterization, the present work provides a structured quality-risk assessment for a low-dose herbal lozenge. The novelty of the study lies in the sequential use of Ishikawa analysis and FMEA, quantitative RPN-based prioritization, predicted residual risk assessment, and formulation of proposed risk mitigation measures within an ICH Q9(R1)-aligned QbD framework.

Overall, direct compression of lozenges containing dry Ajania fruticulosa extract requires enhanced control of both CMAs and CPPs. The proposed control strategy provides a scientific basis for further process development, documentation, technology transfer, and scale-up.

Limitations. FMEA scoring was based on expert judgment, and residual RPN values reflect predicted rather than experimentally confirmed risk reduction. In addition, the reported process ranges should be regarded as preliminary operating ranges rather than a formally established design space.

Conclusion

QbD–based quality risk assessment of the direct compression technology for lozenges containing dry Ajania fruticulosa extract was performed by combining an Ishikawa cause-and-effect diagram with FMEA. Seven priority failure modes were identified, of which segregation of the powder mass during feeding into the hopper zone carried the highest initial risk. Corrective and preventive actions targeting critical material attributes and critical process parameters were predicted to reduce the RPN across all evaluated failure modes, reassigning six of the seven modes to the low-risk zone, whereas segregation remained a medium residual risk requiring continued in-process monitoring. The resulting control strategy provides a scientifically justified basis for managing material and process-related variability during manufacturing. These expert–based predictions should be confirmed experimentally through direct measurement of the corresponding CQA during subsequent process development and scale-up.

References

1. ICH Q8(R2). Pharmaceutical Development. – Geneva: International Council for Harmonisation, 2009. URL: https://database.ich.org/sites/default/files/Q8_R2_Guideline.pdf (accessed: 09.07.2026)

2. ICH Q9(R1). Quality Risk Management. – Geneva: International Council for Harmonisation, 2023. URL: https://www.ema.europa.eu/en/documents/scientific-guideline/international-conference-harmonisation-technical-requirements-registration-pharmaceuticals-human-use-ich-guideline-q9-r1-quality-risk-management-step-5-revision-2_en.pdf (accessed: 09.07.2026)

3. ICH Q10. Pharmaceutical Quality System. – Geneva: International Council for Harmonisation, 2008. URL: https://database.ich.org/sites/default/files/Q10%20Guideline.pdf (accessed: 09.07.2026)

4. Yang S., Hu X., Zhu J., Zheng B., Bi W., Wang X., Wu J., Mi Z., Wu Y. Aspects and implementation of pharmaceutical Quality by Design from conceptual frameworks to industrial applications // Pharmaceutics. – 2025. – Vol. 17, № 5. – Article 623. – DOI: https://doi.org/10.3390/pharmaceutics17050623.

5. Gurski C.S., Luna Lazo R.E., Gelinski M.A., Alves L.A., Sovinski M.S., Sari M.H.M., Borba H.H.L., Tonin F.S., Pontarolo R., Ferreira L.M. Quality by Design applied to pediatric-friendly formulations: a scoping review-based manufacturing process mapping for solid dosage forms // Journal of Drug Delivery Science and Technology. – 2025. – Vol. 108. – Article 106915. – DOI: https://doi.org/10.1016/j.jddst.2025.106915.

6. Simão J., Chaudhary S.A., Ribeiro A.J. Implementation of Quality by Design for development of bilayer tablets // European Journal of Pharmaceutical Sciences. – 2023. – Vol. 184. – Article 106412. – DOI: https://doi.org/10.1016/j.ejps.2023.106412.

7. Casian T., Iurian S., Bogdan C., Rus L., Moldovan M., Tomuta I. Quality by Design for pediatric oral lyophilisates development: risk assessment followed by screening and optimization // Drug Development and Industrial Pharmacy. – 2017. – Vol. 43, № 12. – P. 1932–1944. – DOI: https://doi.org/10.1080/03639045.2017.1350702.

8. Kaljević O., Djuriš J., Djurić Z., Ibrić S. Application of failure mode and effects analysis in Quality by Design approach for formulation of carvedilol compression coated tablets // Journal of Drug Delivery Science and Technology. – 2016. – Vol. 32. – P. 56–63. – DOI: https://doi.org/10.1016/j.jddst.2016.02.004.

9. Nait Bachir Y., Said R.M., Terki N.Z., Antar R., Slamani M., Gharbi D., Foligni R. Biopharmaceutical and pharmacotechnical characterization of plant powder tablets obtained by direct compression process: the case of Atriplex halimus // Applied Sciences. – 2025. – Vol. 15, № 15. – Article 8623. – DOI: https://doi.org/10.3390/app15158623.

10. Pham E.C., Vo Van L., Nguyen C.V., Nguyen Duong N.T., Le Thi T.V. Formulation development, optimization, in vivo antidiabetic effect and acute toxicity of directly compressible herbal tablets containing Merremia tridentata (L.) extract // Journal of Drug Delivery Science and Technology. – 2023. – Vol. 84. – Article 104445. – DOI: https://doi.org/10.1016/j.jddst.2023.104445.

11. Tafere C., Yilma Z., Abrha S., Yehualaw A. Formulation, in vitro characterization and optimization of taste-masked orally disintegrating co-trimoxazole tablet by direct compression // PLOS ONE. – 2021. – Vol. 16, № 3. – Article e0246648. – DOI: https://doi.org/10.1371/journal.pone.0246648.

12. Naman S., Madhavi N., Singh B., Madan J., Baldi A. Implementing risk-based Quality by Design for development and optimization of flavored oral disintegrating mini tablets // Journal of Drug Delivery Science and Technology. – 2021. – Vol. 66. – Article 102799. – DOI: https://doi.org/10.1016/j.jddst.2021.102799.

13. Джакиянов А.М., Ридванов Ч.И., Исаев Ж.Н., Жумашова Г.Т., Бекежанова Т.С., Кулманбетов Р.И., Талгаева Е.В., Туруспаева Ж.Ж., Сакипова З.Б., Сулейменов М.К. Разработка состава и технологии таблеток для рассасывания с экстрактом Ajania fruticulosa (Ledeb.) Poljak с применением концепции Quality by Design // Разработка и регистрация лекарственных средств. – 2026. – Т. 15, № 1. – С. 85–95. – DOI: https://doi.org/10.33380/2305-2066-2026-15-1-2233.

14. Liang J., Lu P., Ning A., Yang Y., Shao Y., Xu J. Chemical constituents from the aerial parts of Ajania fruticulosa // Biochemical Systematics and Ecology. – 2020. – Vol. 92. – Article 104124. – DOI: https://doi.org/10.1016/j.bse.2020.104124.

15. Min X., Chen J., Mu W., Huang J., Huang G., Cao J. A new dimeric sesquiterpenoid from Ajania fruticulosa // Phytochemistry Letters. – 2025. – Vol. 66. – P. 60–65. – DOI: https://doi.org/10.1016/j.phytol.2025.02.001.

16. Джакиянов А.М., Сакипова З.Б., Ибрагимова Л.Н., Ибраева М.Б., Калдыбаева А.М., Енсебаева Г.Б. Фармацевтическая композиция из растительной субстанции с антибактериальным и противогрибковым действием и способ её получения. – Патент РК на полезную модель № 11229. – Бюл. № 40. – 03.10.2025. URL: https://gosreestr.kazpatent.kz/Utilitymodel/Details?docNumber=429851 (accessed:09.07.2026).

17. Liu H.-C., Chen X.-Q., Duan C.-Y., Wang Y.-M. Failure mode and effect analysis using multi-criteria decision making methods: A systematic literature review // Computers & Industrial Engineering. – 2019. – DOI: https://doi.org/10.1016/j.cie.2019.06.055.

18. Ccopa Ibarra L. A., Fontenele Urano de Carvalho E., Durazzo M. Overcoming FMEA shortcomings: Proposal of an adapted risk analysis matrix // Engineering Failure Analysis. – 2024. – Vol. 166. – Article 108796. – DOI: https://doi.org/10.1016/j.engfailanal.2024.108796.

References

- ICH Q8(R2). Pharmaceutical Development. – Geneva: International Council for Harmonisation, 2009. URL: https://database.ich.org/sites/default/files/Q8_R2_Guideline.pdf (accessed: 09.07.2026)

- ICH Q9(R1). Quality Risk Management. – Geneva: International Council for Harmonisation, 2023. URL: https://www.ema.europa.eu/en/documents/scientific-guideline/international-conference-harmonisation-technical-requirements-registration-pharmaceuticals-human-use-ich-guideline-q9-r1-quality-risk-management-step-5-revision-2_en.pdf (accessed: 09.07.2026)

- ICH Q10. Pharmaceutical Quality System. – Geneva: International Council for Harmonisation, 2008. URL: https://database.ich.org/sites/default/files/Q10%20Guideline.pdf (accessed: 09.07.2026)

- Yang S., Hu X., Zhu J., Zheng B., Bi W., Wang X., Wu J., Mi Z., Wu Y. Aspects and implementation of pharmaceutical Quality by Design from conceptual frameworks to industrial applications // Pharmaceutics. – 2025. – Vol. 17, № 5. – Article 623. – DOI: https://doi.org/10.3390/pharmaceutics17050623.

- Gurski C.S., Luna Lazo R.E., Gelinski M.A., Alves L.A., Sovinski M.S., Sari M.H.M., Borba H.H.L., Tonin F.S., Pontarolo R., Ferreira L.M. Quality by Design applied to pediatric-friendly formulations: a scoping review-based manufacturing process mapping for solid dosage forms // Journal of Drug Delivery Science and Technology. – 2025. – Vol. 108. – Article 106915. – DOI: https://doi.org/10.1016/j.jddst.2025.106915.

- Simão J., Chaudhary S.A., Ribeiro A.J. Implementation of Quality by Design for development of bilayer tablets // European Journal of Pharmaceutical Sciences. – 2023. – Vol. 184. – Article 106412. – DOI: https://doi.org/10.1016/j.ejps.2023.106412.

- Casian T., Iurian S., Bogdan C., Rus L., Moldovan M., Tomuta I. Quality by Design for pediatric oral lyophilisates development: risk assessment followed by screening and optimization // Drug Development and Industrial Pharmacy. – 2017. – Vol. 43, № 12. – P. 1932–1944. – DOI: https://doi.org/10.1080/03639045.2017.1350702.

- Kaljević O., Djuriš J., Djurić Z., Ibrić S. Application of failure mode and effects analysis in Quality by Design approach for formulation of carvedilol compression coated tablets // Journal of Drug Delivery Science and Technology. – 2016. – Vol. 32. – P. 56–63. – DOI: https://doi.org/10.1016/j.jddst.2016.02.004.

- Nait Bachir Y., Said R.M., Terki N.Z., Antar R., Slamani M., Gharbi D., Foligni R. Biopharmaceutical and pharmacotechnical characterization of plant powder tablets obtained by direct compression process: the case of Atriplex halimus // Applied Sciences. – 2025. – Vol. 15, № 15. – Article 8623. – DOI: https://doi.org/10.3390/app15158623.

- Pham E.C., Vo Van L., Nguyen C.V., Nguyen Duong N.T., Le Thi T.V. Formulation development, optimization, in vivo antidiabetic effect and acute toxicity of directly compressible herbal tablets containing Merremia tridentata (L.) extract // Journal of Drug Delivery Science and Technology. – 2023. – Vol. 84. – Article 104445. – DOI: https://doi.org/10.1016/j.jddst.2023.104445.

- Tafere C., Yilma Z., Abrha S., Yehualaw A. Formulation, in vitro characterization and optimization of taste-masked orally disintegrating co-trimoxazole tablet by direct compression // PLOS ONE. – 2021. – Vol. 16, № 3. – Article e0246648. – DOI: https://doi.org/10.1371/journal.pone.0246648.

- Naman S., Madhavi N., Singh B., Madan J., Baldi A. Implementing risk-based Quality by Design for development and optimization of flavored oral disintegrating mini tablets // Journal of Drug Delivery Science and Technology. – 2021. – Vol. 66. – Article 102799. – DOI: https://doi.org/10.1016/j.jddst.2021.102799.

- Jakiyanov A.M., Ridvanov Ch.I., Isaev Zh.N., Zhumashova G.T., Bekezhanova T.S., Kulmanbetov R.I., Talgayeva E.V., Turuspayeva Zh.Zh., Sakipova Z.B., Suleimenov M.K. Razrabotka sostava i tekhnologii tabletok dlya rassasyvaniya s ekstraktom Ajania fruticulosa (Ledeb.) Poljak s primeneniem kontseptsii Quality by Design. Razrabotka i registratsiya lekarstvennykh sredstv, 2026, vol. 15, no. 1, pp. 85–95. DOI: 10.33380/2305-2066-2026-15-1-2233. (In Russian).

- Liang J., Lu P., Ning A., Yang Y., Shao Y., Xu J. Chemical constituents from the aerial parts of Ajania fruticulosa. Biochemical Systematics and Ecology, 2020, vol. 92, article 104124. DOI: 10.1016/j.bse.2020.104124.

- Min X., Chen J., Mu W., Huang J., Huang G., Cao J. A new dimeric sesquiterpenoid from Ajania fruticulosa. Phytochemistry Letters, 2025, vol. 66, pp. 60–65. DOI: 10.1016/j.phytol.2025.02.001.

- Jakiyanov A.M., Sakipova Z.B., Ibragimova L.N., Ibrayeva M.B., Kaldybayeva A.M., Ensebaeva G.B. Farmatsevticheskaya kompozitsiya iz rastitelnoi substantsii s antibakterialnym i protivogribkovym deistviem i sposob ee polucheniya. Republic of Kazakhstan utility-model patent No. 11229, Bulletin No. 40, 3 October 2025. Available at: Kazpatent utility model record (accessed: 09.07.2026). (In Russian).

- Liu H.-C., Chen X.-Q., Duan C.-Y., Wang Y.-M. Failure mode and effect analysis using multi-criteria decision making methods: a systematic literature review. Computers & Industrial Engineering, 2019. DOI: 10.1016/j.cie.2019.06.055.

- Ccopa Ibarra L.A., Fontenele Urano de Carvalho E., Durazzo M. Overcoming FMEA shortcomings: proposal of an adapted risk analysis matrix. Engineering Failure Analysis, 2024, vol. 166, article 108796. DOI: 10.1016/j.engfailanal.2024.108796.

ОЦЕНКА РИСКОВ КАЧЕСТВА ТЕХНОЛОГИИ ТАБЛЕТОК ДЛЯ РАССАСЫВАНИЯ С ЭКСТРАКТОМ AJANIA FRUTICULOSA (LEDEB.) POLJAK В РАМКАХ КОНЦЕПЦИИ QUALITY BY DESIGN

А.М. Джакиянов1, З.Б. Сакипова1, З.Б. Алламбергенова1, А. Кадырбай2, Р.И. Кульманбетов1, Г.С. Ибадуллаева1, М.К. Сулейменов¹*

¹ Казахский национальный медицинский университет имени С.Д. Асфендиярова, Республика Казахстан, г. Алматы

² ТОО «Viva Pharm», Республика Казахстан, г. Алматы

*Корреспондирующий автор

Аннотация

Актуальность. Прямое прессование является технологически и экономически обоснованным способом получения таблеток для рассасывания, исключающим стадии влажного гранулирования и сушки. Вместе с тем вариабельность физико-химических и технологических свойств субстанций растительного происхождения повышает вероятность возникновения рисков, способных нарушить воспроизводимость критических показателей качества. Применение риск-ориентированного подхода в соответствии с руководством Международного совета по гармонизации Q9(R1) в рамках концепции Quality by Design является необходимым условием систематической идентификации источников вариабельности и научного обоснования стратегии контроля.

Цель исследования. Идентификация и оценка рисков при разработке технологии прямого прессования таблеток для рассасывания с экстрактом сухим Ajania fruticulosa (Ledeb.) Poljak в рамках концепции Quality by Design.

Материалы и методы. Источники вариабельности систематизированы с применением причинно-следственной диаграммы Исикавы. Количественная оценка рисков выполнена методом анализа видов и последствий отказов. Тяжесть последствий, вероятность возникновения и вероятность обнаружения оценивали по пятибалльной шкале с последующим расчетом приоритетного числа риска. Остаточный риск оценивали после моделирования предложенных мер по снижению рисков.

Результаты. Идентифицированы семь приоритетных режимов отказа. Исходные значения приоритетного числа риска находились в диапазоне 18–48. Наибольшее значение установлено для сегрегации таблеточной массы при подаче в зону питателя. Корректирующие и предупреждающие действия, направленные на критические атрибуты материалов и критические параметры процесса по прогнозной оценке снижают остаточные значения приоритетного числа риска до 6–24. На основании остаточного риска шесть из семи режимов отказа переведены в область низкого риска.

Выводы. Предложенная стратегия контроля технологического процесса формирует научно обоснованную основу для подготовки регламентирующей документации, трансфера технологии и масштабирования процесса прямого прессования таблеток для рассасывания, содержащих экстракт сухой Ajania fruticulosa.

Ключевые слова: таблетки для рассасывания, Ajania fruticulosa, Quality by Design, управление рисками качества, диаграмма Исикавы.

QUALITY BY DESIGN ТҰЖЫРЫМДАМАСЫ АЯСЫНДА AJANIA FRUTICULOSA (LEDEB.) POLJAK ЭКСТРАКТІСІ БАР СОРУ ТАБЛЕТКАЛАРЫН ТЕХНОЛОГИЯСЫНЫҢ САПА ТӘУЕКЕЛДЕРІН БАҒАЛАУ

А.М. Джакиянов¹, З.Б. Сакипова¹, З.Б. Алламбергенова¹, А. Кадырбай², Р.И. Құлманбетов¹, Ғ.С. Ибадуллаева1, М.К. Сүлейменов¹*

¹ С.Д. Асфендияров атындағы Қазақ ұлттық медицина университеті, Қазақстан Республикасы, Алматы қ.

² «Viva Pharm» ЖШС, Қазақстан Республикасы, Алматы қ.

* Хат-хабарға жауапты автор

Аңдатпа

Өзектілігі. Тікелей престеу соруға арналған таблеткаларды алудың технологиялық және экономикалық тұрғыдан негізделген тәсілі болып табылады, өйткені ылғалды түйіршіктеу және кептіру сатыларын болдырмайды. Сонымен бірге өсімдік текті субстанциялардың физика-химиялық және технологиялық қасиеттерінің вариабельділігі сапаның сындарлы көрсеткіштерінің қайталанымдылығын бұзуы мүмкін тәуекелдердің туындау ықтималдығын арттырады. Quality by Design тұжырымдамасы шеңберінде Халықаралық үйлестіру кеңесінің Q9(R1) нұсқаулығына сәйкес тәуекелге бағытталған тәсілді қолдану вариабельділік көздерін жүйелі түрде анықтау және бақылау стратегиясын ғылыми негіздеу үшін қажетті шарт болып табылады.

Зерттеудің мақсаты. Зерттеудің мақсаты Quality by Design тұжырымдамасы шеңберінде Ajania fruticulosa (Ledeb.) Poljak құрғақ экстракты бар соруға арналған таблеткаларды тікелей престеу технологиясын әзірлеу кезінде сапамен байланысты тәуекелдерді анықтау және бағалау болды.

Материалдар мен әдістер. Вариабельділіктің ықтимал көздері Исикава себеп-салдар диаграммасын қолдану арқылы жүйелі түрде анықталып, құрылымдалды. Тәуекелдерді сандық бағалау істен шығу түрлері мен олардың салдарын талдау әдісі арқылы жүргізілді. Салдардың ауырлығы, туындау ықтималдығы және анықталу ықтималдығы бес балдық шкала бойынша бағаланып, кейін тәуекелдің басымдық саны есептелді. Қалдық тәуекел ұсынылған тәуекелдерді төмендету шараларын модельдеуден кейін бағаланды.

Нәтижелер. Жеті басым істен шығу режимі анықталды. Тәуекелдің бастапқы басымдық саны 18–48 аралығында болды. Ең жоғары мән ұнтақ қоспасының қоректендіру аймағына берілуі кезінде сегрегацияға тән болды. Болжамды бағалау бойынша, материалдардың критикалық атрибуттары мен процестің критикалық параметрлерін бақылауға бағытталған ұсынылған тәуекелдерді төмендету шаралары қалдық тәуекелдің басымдық санын 6–24 аралығына дейін азайтуға мүмкіндік берді. Қалдық тәуекелді бағалау нәтижесінде жеті істен шығу режимінің алтауы төмен тәуекел аймағына ауыстырылды.

Қорытынды. Ұсынылған технологиялық процесті бақылау стратегиясы Ajania fruticulosa құрғақ экстракты бар соруға арналған таблеткаларды тікелей престеу процесі бойынша регламенттік құжаттаманы дайындау, технологияны трансферлеу және масштабтау үшін ғылыми негізделген негіз қалыптастырады.

Түйін сөздер: соруға арналған таблеткалар, Ajania fruticulosa, Quality by Design, сапа тәуекелдерін басқару, Исикава диаграммасы.

ОБ АВТОРАХ

Джакиянов Амангельды Муратович – ассистент-профессор кафедры биотехнологии и общей химической технологии Школы Фармации, НАО «Казахский национальный медицинский университет имени С.Д. Асфендиярова», Казахстан, Алматы, e-mail: dzhakiyanov.a@kaznmu.kz, ORCID: https://orcid.org/0000-0001-5239-9429.

Сакипова Зуриядда Бектемировна – д.фарм.н., профессор, декан Школы Фармации, НАО «Казахский национальный медицинский университет имени С.Д. Асфендиярова», Казахстан, Алматы, e-mail: sakipova.z@kaznmu.kz, ORCID: https://orcid.org/0000-0003-1400-1971.

Алламбергенова Зоя Бакбергенкызы – PhD, ассоциированный профессор без ученого звания кафедры инженерных дисциплин, НАО «Казахский национальный медицинский университет имени С.Д. Асфендиярова», Казахстан, Алматы, e-mail: allambergenova.z@kaznmu.kz, ORCID: https://orcid.org/0000-0002-0801-0508.

Кадырбай Аршын – руководитель отдела фармацевтических разработок ТОО «Viva Pharm», Казахстан, Алматы, e-mail: akadyrbai@vivapharm.kz, ORCID: https://orcid.org/0000-0002-5288-6742.

Кульманбетов Руслан Исатаевич – к.м.н., ассоциированный профессор без ученого звания, заведующий кафедрой ортодонтии, НАО «Казахский национальный медицинский университет имени С.Д. Асфендиярова», Казахстан, Алматы, e-mail: kulmanbetov.r@kaznmu.kz, ORCID: https://orcid.org/0000-0002-9631-0112.

Ибадуллаева Ғалия Сәруарқызы – PhD, ассоциированный профессор, профессор кафедры фармацевтической технологии Школы Фармации, НАО «Казахский национальный медицинский университет имени С.Д. Асфендиярова», Казахстан, Алматы, e-mail: arujan-d@mail.ru, ORCID: https://orcid.org/0000-0002-1268-9867.

Сулейменов Марат Камытбекович – корреспондирующий автор, к.б.н., ассоциированный профессор без ученого звания кафедры биотехнологии и общей химической технологии, НАО «Казахский национальный медицинский университет имени С.Д. Асфендиярова», Казахстан, Алматы, e-mail: suleimenov.m@kaznmu.kz, ORCID: https://orcid.org/0009-0005-6902-6262.

АВТОРЛАР ТУРАЛЫ

Джакиянов Амангельды Муратович – Фармация мектебінің биотехнология және жалпы химиялық технология кафедрасының ассистент-профессоры, С.Д. Асфендияров атындағы Қазақ ұлттық медицина университеті, Қазақстан, Алматы, e-mail: dzhakiyanov.a@kaznmu.kz, ORCID: https://orcid.org/0000-0001-5239-9429.

Сакипова Зуриядда Бектемировна – фармация ғылымдарының докторы, профессор, Фармация мектебінің деканы, С.Д. Асфендияров атындағы Қазақ ұлттық медицина университеті, Қазақстан, Алматы, e-mail: sakipova.z@kaznmu.kz, ORCID: https://orcid.org/0000-0003-1400-1971.

Алламбергенова Зоя Бақбергенқызы – PhD, инженерлік пәндер кафедрасының ғылыми атағы жоқ қауымдастырылған профессоры, С.Д. Асфендияров атындағы Қазақ ұлттық медицина университеті, Қазақстан, Алматы, e-mail: allambergenova.z@kaznmu.kz, ORCID: https://orcid.org/0000-0002-0801-0508.

Кадырбай Аршын – «Viva Pharm» ЖШС фармацевтикалық әзірлемелер бөлімінің жетекшісі, Қазақстан, Алматы, e-mail: akadyrbai@vivapharm.kz, ORCID: https://orcid.org/0000-0002-5288-6742.

Құлманбетов Руслан Исатайұлы – м.ғ.к., ғылыми атағы жоқ қауымдастырылған профессор, ортодонтия кафедрасының меңгерушісі, С.Д. Асфендияров атындағы Қазақ ұлттық медицина университеті, Қазақстан, Алматы, e-mail: kulmanbetov.r@kaznmu.kz, ORCID: https://orcid.org/0000-0002-9631-0112.

Ибадуллаева Ғалия Сәруарқызы – PhD, қауымдастырылған профессор, фармацевтикалық технология кафедрасының профессоры, Фармация мектебі, «С.Д. Асфендияров атындағы Қазақ ұлттық медицина университеті» КеАҚ, Қазақстан, Алматы, e-mail: arujan-d@mail.ru, ORCID: https://orcid.org/0000-0002-1268-9867.

Сүлейменов Марат Камытбекұлы – хат-хабарға жауапты автор, биология ғылымдарының кандидаты, биотехнология және жалпы химиялық технология кафедрасының ғылыми атағы жоқ қауымдастырылған профессоры, С.Д. Асфендияров атындағы Қазақ ұлттық медицина университеті, Қазақстан, Алматы, e-mail: suleimenov.m@kaznmu.kz, ORCID: https://orcid.org/0009-0005-6902-6262.

ABOUT AUTHORS

Jakiyanov Amangeldy Muratovich – Assistant Professor, Department of Biotechnology and General Chemical Technology, School of Pharmacy, S.D. Asfendiyarov Kazakh National Medical University, Almaty, Kazakhstan, e-mail: dzhakiyanov.a@kaznmu.kz, ORCID: https://orcid.org/0000-0001-5239-9429.

Sakipova Zuriadda Bektemirovna – Doctor of Pharmaceutical Sciences, Professor, Dean of the School of Pharmacy, S.D. Asfendiyarov Kazakh National Medical University, Almaty, Kazakhstan, e-mail: sakipova.z@kaznmu.kz, ORCID: https://orcid.org/0000-0003-1400-1971.

Allambergenova Zoya Bakbergenqyzy – PhD, Associate Professor without academic title, Department of Engineering Disciplines, S.D. Asfendiyarov Kazakh National Medical University, Almaty, Kazakhstan, e-mail: allambergenova.z@kaznmu.kz, ORCID: https://orcid.org/0000-0002-0801-0508.

Kadyrbay Arshyn – Head of Pharmaceutical Development, Viva Pharm LLP, Almaty, Kazakhstan, e-mail: akadyrbai@vivapharm.kz, ORCID: https://orcid.org/0000-0002-5288-6742.

Kulmanbetov Ruslan Isatayevich – Candidate of Medical Sciences, Associate Professor without academic title, Head of the Department of Orthodontics, S.D. Asfendiyarov Kazakh National Medical University, Almaty, Kazakhstan, e-mail: kulmanbetov.r@kaznmu.kz, ORCID: https://orcid.org/0000-0002-9631-0112.

Ibadullayeva Galiya Saruarkyzy – PhD, Associate Professor, Professor of the Department of Pharmaceutical Technology, School of Pharmacy, S.D. Asfendiyarov Kazakh National Medical University, Almaty, Kazakhstan, e-mail: arujan-d@mail.ru, ORCID: https://orcid.org/0000-0002-1268-9867.

Suleimenov Marat Kamytbekovich – Corresponding author, Candidate of Biological Sciences, Associate Professor without academic title, Department of Biotechnology and General Chemical Technology, S.D. Asfendiyarov Kazakh National Medical University, Almaty, Kazakhstan, e-mail: suleimenov.m@kaznmu.kz, ORCID: https://orcid.org/0009-0005-6902-6262.

Authors’ contributions. A.M. Jakiyanov conceived and designed the study and performed the main stages of the research. Z.B. Sakipova, Z.B. Allambergenova, G.S. Ibadullayeva and A. Kadyrbay contributed to the experimental work and data collection. M.K. Suleimenov and R.I. Kulmanbetov contributed to the interpretation of the results and critical revision of the manuscript. All authors participated in manuscript preparation.

Conflict of interest. The authors declare no conflict of interest.

Funding. This research received no external funding.

Declaration of Generative AI and AI-assisted technologies. During the preparation of this work, the authors used AI-assisted technologies solely to improve the English translation and enhance the grammatical and stylistic quality of the manuscript. After using this tool, the authors reviewed and edited the content as needed and take full responsibility for the accuracy and scientific integrity of the final publication.

All authors have approved the final version of the article and are responsible for its content.